# RELEASE NOTES

## Mercury 310 Firmware Release

## Version H7.03

| Release Date: | 11-01-2024 (November 1st, 2024) |
| --- | --- |
| File names: | USB file for field upgrades: Mercury310update_H7_03.bin |
| Field Programming Device: | USB Disk |

### New Features

- User defined slowdown points.

  - How to set open or close slowdown points:

    - Limits must be learned prior to adjusting slowdown points.

    - Turn the UI knob to LRN MOTOR 1 to adjust motor 1 slowdown points or turn to LRN MOTOR 2 to adjust motor 2 slowdown points.

    - Ensure that “L1” or “L2” is shown on the display (adjust UI knob position if needed):

      - Press the open or close button to adjust the associated slowdown point:

        - The associated limit LED will flash to indicate whether the open or close slowdown point is currently being adjusted.

        - The display will show the current slowdown point.

        - Use the arrow buttons to adjust the slowdown point.

        - To save the new slowdown point, press the same open or close button used to adjust either the open or close slowdown point.

          - Slowdown point is not automatically saved.

          - User must press the same open or close button to save.

        - To cancel, press the stop button.

          - NOTE:  The stop button causes the display to no longer display UI knob associated settings (i.e., LRN MOTOR 1, Force, Close Timer, etc.).

- Juno motor support.

### Changes/Enhancements

- Updated IES threshold during acceleration for linear actuators.