# RELEASE NOTES

## Mercury 310 Firmware Release

## Version H7.05

| Release Date: | 11-06-2025 (November 06, 2025) |
| --- | --- |
| File names: | USB file for field upgrades: Mercury310update_H7_05Build3_11_06_2025.bin |
| Field Programming Device: | USB Disk |

### New Features

- None.

### Changes/Enhancements

- Updated when user defined slowdown points can be set by the user.

  - Slowdown point range limits:

    - Open slowdown range from 60% - 97%.

    - Close slowdown range from 3% - 40%.

  - Slowdown points can be set before learning.

    - Previous releases required the gate to be learned.

  - Slowdown points are only reset during a factory reset.

  - How to set open or close slowdown points:

    - Turn the UI knob to LRN MOTOR 1 to adjust motor 1 slowdown points or turn to LRN MOTOR 2 to adjust motor 2 slowdown points.

    - Press the open or close button to adjust the associated slowdown point:

      - The associated limit LED will flash to indicate whether the open or close slowdown point is currently being adjusted.

      - The display will show the current slowdown point.

      - Use the arrow buttons to adjust the slowdown point.

      - To save the new slowdown point, press the same open or close button used to adjust either the open or close slowdown point.

        - Slowdown point is not automatically saved.

        - User must press the same open or close button to save.

      - To cancel, press the stop button.

- Updated maximum run time:

  - Learn mode run time timeout set to 2 minutes.

  - Run time timeout when position is unknown is set to 2 minutes.

  - Run time timeout when position is known is set to 1 minute for swing gates and 1min 30s for slide gates (Juno).

- Updated default deceleration points for Juno to prevent the gate from running past the limit.

### Known Issues

- USB Drive causes odd and unpredictable gate behavior.

  - Odd and unpredictable behavior includes the gate running past the limit, the gate starting to open/close without a user input, and so forth.

  - If odd gate behavior is observed while a USB drive is installed to gather logging data, then remove the USB drive immediately.  A power cycle of the Mercury controller is recommended after removing the USB drive.

  - Use a different USB drive for firmware upgrades and data logging.

### Upgrade Instructions

- Place the firmware upgrade file on a USB drive (FAT file system).

  - If the firmware upgrade file is within a .zip file, then extract the firmware upgrade file (binary file format .bin) and copy to the USB drive.

  - Do not place the firmware upgrade file in a folder.  It must be on the root of the USB drive.

- On the Mercury 310 controller, press and hold the firmware update button (located next to the USB port) until “bL” is displayed to enter the bootloader.

- Insert the USB drive containing the firmware upgrade file.

- The upgrade should be finished within 5 seconds and will automatically reset and show the new firmware version on the display during startup.

  - It is recommended to leave the USB drive inserted after upgrading to enable data logging.  Logs are stored on the USB drive and can provide valuable information to aid in troubleshooting at installation sites.

    - The USB drive can be safely removed when the USB LED is off.